EmuXY and KML 2.0

Introduction

This document explains in detail the KML 2.0 scripting files. KML, Keyboard Mapping Language, started of as a script that Emu48 used to map keyboard buttons with buttons of the emulator, now it has become much more. KML 2.0, also used by Emu10, Emu28, Emu32 and Emu42, now makes it possible to change many advanced options of the way the emulator looks.

Index

Basics	Page 1

Global	Page 2

Background	Page 5

LCD	Page 6

Digit	Page 8

Annunciator	Page 10

Button	Page 11

OutIn Codes	Page 13

Scancode	Page 21

Conclusion	Page 22

Basics

A KML script file is a text file that the emulators use to set its screen layout and keyboard layout. A custom bitmap file can be created to be the “faceplate” of the calculator. KML defines the size and position of the screen, buttons, annunciators, and the Emu48 window. The “#” character can be used to add remarks to a line. The following sections describe each main block of the KML script.

Global

The global section of the KML file tells the bitmap, title of the KML, and the Calculator hardware and the model. The following is the syntax.

Global

Title STRING

Author STRING

Hardware STRING

Model STRING

Class INTEGER

Rom STRING

Patch STRING

Bitmap STRING

Print STRING

Debug INTEGER

End

Title is the title of the KML file that the emulator will use. This is not the filename.

Example:

Title “John’s Gx”

Author is the author of the KML file.

Example:

Author “John Doe”

Hardware is the emulated calculator hardware. This definition is optional on Emu48. The parameter is necessary to separate the KML scripts from each emulator. Valid entries are

Emu10:

Hardware “Bert”

Emu28:

Hardware “Centipede”

Emu32:

Hardware “Sacajawea”

Emu42:

Hardware “Lewis”

Emu48:

Hardware “Yorke”

Model is the model of your emulated calculator inside the hardware family. The model entries differ from hardware to hardware. This setting groups KML files of the same hardware and ROM together. When switching between KML scripts, it is the first character of the parameter that is checked against the type of the current document (which was set by the Model command in the KML file with which the document was created). Valid entries are:

Emu10:

“E” = HP10B

“F” = HP20S

“M” = HP21S

Emu28:

“P” = HP28C

Emu32:

“M” = HP14B

“N” = HP32SII

Emu42:

“T” = HP17B

“U” = HP17BII

“Y” = HP19BII

“M” = HP27S

“O” = HP28S

“D” = HP42S

Emu48:

“6” = HP38G (64KB RAM)

“A” = HP38G

“E” = HP39G or HP40G

“G” = HP48G, HP48G+ or HP48GX

“S” = HP48S or HP48SX

“X” = HP49G

Example:

Model “G”

Class is used for different calculators base on the same ROM for further distinction. For other models than listed below this argument can be omitted. Valid entries are:

Emu42:

“32” = HP42S with 32KB RAM

Emu48:

“39” = HP39G

“40” = HP40G

Example:

Class 39

Rom is the name of your ROM file.

Example:

Rom “ROM.48G”

Patch is the name of a ROM patch file. Any file with correct syntax can be used as a patch file. Multiple Patch lines can be used, and all files will be loaded in the order in which they are declared. Important: They MUST appear after the Rom command.

Example:

Patch “BEEP.EXT”

Bitmap is the bitmap file that will be the “faceplate”

Example:

Bitmap “mygx.bmp”

Print will display a string when the KML file is loaded. You can have as many of these as necessary. They are more effective if you put them before the other settings.

Example:

Print “My homepage is at http://www.gulftel.com/~pattersc/emu48/”

Debug sets the emulator to return an information message box telling the scancode number of the keyboard key pressed when the scancode number isn’t defined. Valid entries are 0 or 1. One is on and Zero is off.

Example:

Debug 1

Background

This section sets where the background bitmap will be placed and the size of the emulator window.

Background

Vga INTEGER

Topbar INTEGER

Offset INTEGER INTEGER

Size INTEGER INTEGER

End

Offset is how far the top left corner of the bitmap will be offset in the window. Most of the time you should set this to 0 0. The first integer is the amount of pixels to the right and the second is that amount of pixels down.

Example:

Offset 0 0

Size sets the size of the emulator window in pixels. The first integer is the width and the second is the height. This is the width of the window, not the bitmap. It is usually smaller that the bitmap size.

Example:

Size 302 485

Vga is a special command only for the Pocket PC versions of Emu32, Emu42 and Emu48 to enable/disable the use of VGA screen resolution on a VGA capable Pocket PC. Valid entries are 0 or 1. One is the VGA mode with 640x480 pixel and Zero is the QVGA mode with 320x240 pixel.

Example:

Vga 1

Topbar is a special command only for the Pocket PC versions of Emu32, Emu42 and Emu48 to enable/disable the topbar of the screen. Valid entries are 0 or 1. One is enabled and Zero is disabled.

Example:

Topbar 0

LCD

This section set the size, position and contrast setting of the LCD screen.

Lcd

Zoom INTEGER

Vertical INTEGER

Offset INTEGER INTEGER

Color INTEGER INTEGER INTEGER INTEGER

End

Zoom is the size of the pixels in the LCD screen. This command is only valid for Emu28, Emu42 and Emu48 and will be ignored on Emu10 and Emu32. Valid entries are 1, 2, 3 or 4. Zoom factor two is usually used by the dot matrix LCD types emulated by Emu28, Emu42 and Emu48.

Example:

Zoom 2

Offset sets the position of the LCD screen. The first integer is the amount of pixels right and the second is the amount of pixels down.

Example:

Offset 20 20

Color sets the color of the LCD display for different contrast settings. The first Integer is the contrast setting for each possible value. The number of settings depend on the hardware type.

Emu10:

0-7 = 0 is lightest, 7 is darkest

Emu32:

0-15 = foreground colors (Pixel on), 0 is lightest, 15 is darkest

16-31 = background colors (Pixel off), 16 corresponds to 0, 17 to 1, …

unused number at background colors = transparent

Emu28, Emu42, Emu48:

0-31 = foreground colors (Pixel on), 0 is lightest, 15 is darkest

32-63 = background colors (Pixel off), 32 corresponds to 0, 33 to 1, …

unused number at background colors = use color defined by setting 0

You should include one line for every foreground color setting. But the calculator Rom bounds the contrast setting with the keyboard to useful values. The HP48S/SX Rom use values between 3 and 19, the HP48G/GX Rom use values between 9 and 24. Remember this when you write the color table please. The other three integers are the RGB numbers (Red, Green, Blue). The RGB numbers range from 0 to 255. The background color number for the corresponding foreground color is always calculated by adding the first background number setting to the color value. If the background color for the contrast setting isn't defined, color 0 is used by the emulators Emu28, Emu42 and Emu48 and transparent mode by Emu32.

Example:

Color 0 255 255 255

Color 1 220 220 220

…

Color 30 10 10 10

Color 31 0 0 0

Color 32 255 255 255	# optional background color for contrast setting

Vertical is a special command only for the Pocket PC versions of Emu42 and Emu48 to rotate the display by 90˚ clock or anticlockwise to allow skins in landscape mode. Valid entries are 0 for portrait, 1 for anticlockwise and 2 for clockwise rotated landscape mode. The default setting is portrait mode.

Example:

Vertical 1

Digit

This section is only valid for Emu10 and Emu32 and describes the (alpha-) numeric part of the LCD screen.Emu10 and Emu32 use different methods for creating a numeric value.

Digit

Offset INTEGER INTEGER

Size INTEGER INTEGER

Distance INTEGER

Bitmap STRING

End

Emu10:

The low-end Pioneer series use a classic 7 segment LCD. With the first nine annunciators (segments ‘a’ to ‘g’, ‘dp’ and ‘cm’) one digit is described.

The commands Offset and Size are unused.

Distance is the distance in pixel between the beginnings of each digit, so only one description for a digit is necessary.

Example:

Distance 18		# distance between digits

Bitmap is an optional command for including an external annunciator bitmap instead of using the annunciators in the background bitmap.

Example:

Bitmap “mylcd.bmp”

Emu32:

The mid range Pioneer series use a 5x7 pixel dot matrix for each digit.

Size is the size of one pixel in the LCD screen. Width Height.

Example:

Size 3 4		# size of LCD pixel

Size is the Size of the annunciator in pixels. Width Height.

Example:

Size 16 11

Offset is the position of the first digit inside the display area.

Example:

Offset 17 13		# position of 1st digit

Distance set the distance in pixel between the beginnings of each digit.

Example:

Distance 21		# distance between digits

Bitmap is an optional command for including an external annunciator bitmap instead of using the annunciators in the background bitmap.

Example:

Bitmap “mylcd.bmp”

Annunciator

Annunciators are the 23 (Emu10), 60 (Emu32), six (Emu48) and seven (Emu28, Emu42) status icons on the screen. You must specify one of these blocks for each annunciator.

Annunciator INTEGER

Size INTEGER INTEGER

Offset INTEGER INTEGER

Down INTEGER INTEGER

End

Annunciator tells which annunciator you are setting. Valid entries are emulator depending and can vary from 1 through 6, 7, 23 or 60. The annunciator symbol itself depends on the specific LCD of the calculator. Refer to existing KML scripts for getting the symbol of each annunciator.

Example:

Annunciator 1

Size is the size of the annunciator in pixels. Width Height.

Example:

Size 16 11

Offset is the position that the annunciator will be displayed. This is the blank part.

Example:

Offset 61 4

Down is the position of the annunciator in the bitmap when it is on. There are some restrictions on the different emulators. On Emu10 and Emu32 the annunciators must be drawn in black on a white background. On Emu28 and Emu42 the pixel at the Down position must contain the background color, this is necessary for masking operation. Emu48 don’t mask the background color, here the annunciator must have the background color of the target area.

Example:

Down 16 485

Button

Button sets the position of the button and what the button does.

Button INTEGER

Type INTEGER

Size INTEGER INTEGER

Offset INTEGER INTEGER

Down INTEGER INTEGER

OutIn INTEGER INTEGER

Virtual

NoHold

OnUp

<Commands>

End

OnDown

<Commands>

End

End

Button tells the number of the button you are creating. This can be any number. Most of the time this is set to a number that represents the row and column of the button. Any number can be used, but only 256 buttons can be declared. If more buttons are declared, only the first 256 will be used, and the others will be ignored.

Example:

Button 11

Type tells how the button will behave when it is pressed. Valid entries are 0, 1, 2, 3, 4, or 5. Type 0 sets it to recognize the Up and Down option. Type 1 sets it to only recognize the Up setting and use a default button effect. Type 2 makes the button look the same when pressed. Type 3 is used for the LCD screen and will invert that area. Type 4 is also used for the LCD screen and it will show the bitmap behind it while pressed. Type 5 draws a transparent circle with a diameter of the size height parameter into the middle of the button area.

Example:

Type 0

Size sets the size of the button. Width height.

Example:

Size 36 25

Offset sets the position of the button. This is when it is not pressed. Pixels right and pixels down.

Example:

Offset 8 158

Down sets the picture of the down button. This is only needed if type 0 is set. Pixels right and pixels down.

Example:

Down 302 25

OutIn tells Emu48 which button is being pressed. A table of OutIn codes are on the next page of this document.

Example:

OutIn 1 16

Virtual mode should be used for buttons that duplicate the function of a HP48 button. This type of button cannot be held down. A OnDown event is generated when the button pressed. A OnUp event is generated when the button is released.

Example:

Virtual

NoHold will keep a button from sticking in the down position. It only generates an OnUp event.

Example:

NoHold

OnDown and OnUp are used to carryout scancode commands when you press a button. Any Scancode command (see page 21) can be used. OnDown is when the button is pressed down. OnUp is when the button is released.

Example:

OnDown

Press 81

Press 63

End

OnUp

Release 63

Release 81

End

OutIn Codes HP10B

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| N | I/YR | PV | PMT | FV | SUM+ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| % | RCL | CFj | CST | PRC | MAR |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| INPUT |  | ->M | RM | M+ | <- |
| 4 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| +/- |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| K |  | 4 | 5 | 6 | * |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| SHIFT |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| C |  | 0 | . | = | + |
| 0 32768 |  | 3 1 | 2 1 | 1 1 | 0 1 |

OutIn Codes HP14B

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| N | I/YR | PV | PMT | FV | SUM+ |
| 0 16 | 1 16 | 2 16 | 5 15 | 4 16 | 3 16 |
| STO | RCL | CST | PRC | MAR | STAT |
| 0 1 | 1 1 | 2 1 | 5 1 | 4 1 | 3 1 |
| INPUT |  | +/- | ( | ) | <- |
| 1 4 |  | 2 4 | 5 4 | 4 4 | 3 4 |
| % |  | 7 | 8 | 9 | / |
| 0 2 |  | 2 2 | 5 2 | 4 2 | 3 2 |
| v |  | 4 | 5 | 6 | * |
| 0 8 |  | 2 8 | 5 8 | 4 8 | 3 8 |
| SHIFT |  | 1 | 2 | 3 | - |
| 0 32 |  | 2 32 | 5 32 | 4 32 | 3 32 |
| C |  | 0 | . | = | + |
| 0 32768 |  | 2 64 | 5 54 | 4 64 | 3 64 |

OutIn Codes HP17B / HP17BII

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| ^ | ^ | ^ | ^ | ^ | ^ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| STO | RCL | % | DSP | PRT | EXIT |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| INPUT |  | +/- | ( | ) | <- |
| 4 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| ^ |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| v |  | 4 | 5 | 6 | * |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| SHIFT |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| C |  | 0 | . | = | + |
| 0 32768 |  | 3 1 | 2 1 | 1 1 | 0 1 |

OutIn Codes HP19BII

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| A | B | C | D | E | F |
| 6 256 | 6 2048 | 6 64 | 6 32 | 6 1 | 6 1024 |
| G | H | I | J | K | L |
| 5 256 | 5 2048 | 5 64 | 5 32 | 5 1 | 5 1024 |
| M | N | O | P | Q | R |
| 4 256 | 4 2048 | 4 64 | 4 32 | 4 1 | 4 1024 |
| S | T | U | V | W | X |
| 3 256 | 3 2048 | 3 64 | 4 32 | 4 1 | 4 1024 |
| Y | Z | ? | $ | # | : |
| 2 256 | 2 2048 | 2 64 | 2 32 | 2 1 | 2 1024 |
| SPACE |  | INS | DEL | <- | -> |
| 1 2048 |  | 1 64 | 1 32 | 1 1 | 1 1024 |

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| 7 2 | 7 128 | 7 512 | 7 4 | 7 8 | 7 16 |
| SHIFT | STO | RCL | DISP | PRNT | EXIT |
| 6 2 | 6 128 | 6 512 | 6 4 | 6 8 | 6 16 |
| INPUT |  | +/- | ( | ) | <- |
| 6 128 |  | 5 512 | 5 4 | 5 8 | 5 16 |
| ^ |  | 7 | 8 | 9 | / |
| 4 2 |  | 4 128 | 4 512 | 4 4 | 4 8 |
| v |  | 4 | 5 | 6 | * |
| 3 2 |  | 3 128 | 3 512 | 3 4 | 3 8 |
| % |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| ON |  | O | . | = | + |
| 0 32768 |  | 1 128 | 1 512 | 1 4 | 1 8 |

OutIn Codes HP20S / HP21S

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| SQR(X) | e^X | LN | Y^X | 1/X | SUM+ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| STO | RCL | SIN | COS | TAN | R/S |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| INPUT |  | +/- | ( | ) | <- |
| 4 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| XEQ |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| <-\ |  | 4 | 5 | 6 | * |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| /-> |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| C |  | 0 | . | = | + |
| 0 32768 |  | 3 1 | 2 1 | 1 1 | 0 1 |

OutIn Codes HP27S

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| ^ | ^ | ^ | ^ | ^ | ^ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| STO | RCL | SIN | COS | TAN | EXIT |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| INPUT |  | +/- | ( | ) | <- |
| 4 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| ^ |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| v |  | 4 | 5 | 6 | * |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| SHIFT |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| C |  | 0 | . | = | + |
| 0 32768 |  | 3 1 | 2 1 | 1 1 | 0 1 |

OutIn Codes HP28C

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| A | B | C | D | E | F |
| 6 16 | 6 32 | 6 2048 | 6 1024 | 6 512 | 6 128 |
| G | H | I | J | K | L |
| 5 16 | 5 32 | 5 2048 | 5 1024 | 5 512 | 5 128 |
| M | N | O | P | Q | R |
| 4 16 | 4 32 | 4 2048 | 4 1024 | 4 512 | 4 128 |
| S | T | U | V | W | X |
| 3 16 | 3 32 | 3 2048 | 3 1024 | 3 512 | 3 128 |
| Y | Z | # | { | [ | ( |
| 2 16 | 2 32 | 2 2048 | 2 1024 | 2 512 | 2 128 |
| SPACE |  | << | = | LC | ALPHA |
| 1 32 |  | 1 2048 | 1 1024 | 1 512 | 1 128 |

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| INS | DEL | ^ | v | < | > |
| 7 256 | 7 64 | 7 8 | 7 1 | 7 2 | 7 4 |
| SHIFT | <^v> | TRIG | SOLV | USER | NEXT |
| 6 256 | 6 64 | 6 8 | 6 1 | 6 2 | 6 4 |
| ENTER |  | CHS | EEX | DROP | <- |
| 5 64 |  | 5 8 | 5 1 | 5 2 | 5 4 |
| ' |  | 7 | 8 | 9 | / |
| 4 256 |  | 4 64 | 4 8 | 4 1 | 4 2 |
| STO |  | 4 | 5 | 6 | * |
| 3 256 |  | 3 64 | 3 8 | 3 1 | 3 2 |
| EVAL |  | 1 | 2 | 3 | - |
| 2 256 |  | 2 64 | 2 8 | 2 1 | 2 2 |
| ON |  | O | . | , | + |
| 0 32768 |  | 1 64 | 1 8 | 1 1 | 1 2 |

OutIn Codes HP28S

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| A | B | C | D | E | F |
| 6 256 | 6 2048 | 6 64 | 6 32 | 6 1 | 6 1024 |
| G | H | I | J | K | L |
| 5 256 | 5 2048 | 5 64 | 5 32 | 5 1 | 5 1024 |
| M | N | O | P | Q | R |
| 4 256 | 4 2048 | 4 64 | 4 32 | 4 1 | 4 1024 |
| S | T | U | V | W | X |
| 3 256 | 3 2048 | 3 64 | 4 32 | 4 1 | 4 1024 |
| Y | Z | # | { | [ | ( |
| 2 256 | 2 2048 | 2 64 | 2 32 | 2 1 | 2 1024 |
| SPACE |  | << | = | LC | ALPHA |
| 1 2048 |  | 1 64 | 1 32 | 1 1 | 1 1024 |

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| INS | DEL | ^ | v | < | > |
| 7 2 | 7 128 | 7 512 | 7 4 | 7 8 | 7 16 |
| SHIFT | <^v> | TRIG | SOLV | USER | NEXT |
| 6 2 | 6 128 | 6 512 | 6 4 | 6 8 | 6 16 |
| ENTER |  | CHS | EEX | DROP | <- |
| 6 128 |  | 5 512 | 5 4 | 5 8 | 5 16 |
| ' |  | 7 | 8 | 9 | / |
| 4 2 |  | 4 128 | 4 512 | 4 4 | 4 8 |
| STO |  | 4 | 5 | 6 | * |
| 3 2 |  | 3 128 | 3 512 | 3 4 | 3 8 |
| EVAL |  | 1 | 2 | 3 | - |
| 2 2 |  | 2 128 | 2 512 | 2 4 | 2 8 |
| ON |  | O | . | , | + |
| 0 32768 |  | 1 128 | 1 512 | 1 4 | 1 8 |

OutIn Codes HP32SII

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| SQr(X) | e^X | LN | Y^X | 1/X | SUM+ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| STO | RCL | Rv | SIN | COS | TAN |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| ENTER |  | X<>Y | +/- | E | <- |
| 4 1 |  | 3 1 | 2 1 | 1 1 | 0 1 |
| XEQ |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| <-\ |  | 4 | 5 | 6 | * |
| 5 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| /-> |  | 1 | 2 | 3 | - |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| EXIT |  | 0 | . | R/S | + |
| 0 32768 |  | 3 2 | 2 2 | 1 2 | 0 2 |

OutIn Codes HP42S

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| SUM+ | 1/X | SQR(X) | LOG | LN | XEQ |
| 5 64 | 4 64 | 3 64 | 2 64 | 1 64 | 0 64 |
| STO | RCL | Rv | SIN | COS | TAN |
| 5 32 | 4 32 | 3 32 | 2 32 | 1 32 | 0 32 |
| ENTER |  | X<>Y | +/- | E | <- |
| 4 16 |  | 3 16 | 2 16 | 1 16 | 0 16 |
| ^ |  | 7 | 8 | 9 | / |
| 5 8 |  | 3 8 | 2 8 | 1 8 | 0 8 |
| v |  | 4 | 5 | 6 | * |
| 5 4 |  | 3 4 | 2 4 | 1 4 | 0 4 |
| SHIFT |  | 1 | 2 | 3 | - |
| 5 2 |  | 3 2 | 2 2 | 1 2 | 0 2 |
| EXIT |  | 0 | . | R/S | + |
| 0 32768 |  | 3 1 | 2 1 | 1 1 | 0 1 |

OutIn Codes HP48SX (HP48GX / HP38G)

| Key |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |
| A | B | C | D | E | F |
| 1 16 | 8 16 | 8 8 | 8 4 | 8 2 | 8 1 |
| G | H | I | J | K | L |
| 2 16 | 7 16 | 7 8 | 7 4 | 7 2 | 7 1 |
| M | N | O | P | Q | R |
| 0 16 | 6 16 | 6 8 | 6 4 | 6 2 | 6 1 |
| S | T | U | V | W | X |
| 3 16 | 5 16 | 5 8 | 5 4 | 5 2 | 5 1 |
| ENTER |  | Y | Z | DEL | DROP |
| 4 16 |  | 4 8 | 4 4 | 4 2 | 4 1 |
| A |  | 7 | 8 | 9 | / |
| 3 32 |  | 3 8 | 3 4 | 3 2 | 3 1 |
| <-\ |  | 4 | 5 | 6 | * |
| 2 32 |  | 2 8 | 2 4 | 2 2 | 2 1 |
| /-> |  | 1 | 2 | 3 | - |
| 1 32 |  | 1 8 | 1 4 | 1 2 | 1 1 |
| ON |  | 0 | . | SPC | + |
| 0 32768 |  | 0 8 | 0 4 | 0 2 | 0 1 |

OutIn Codes HP49G (HP39G / HP40G)

| Key |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OutIn |  |  |  |  |  |  |  |  |  |  |  |
| A | B |  | C |  | D |  | E |  |  | F |  |
| 5 1 | 5 2 |  | 5 4 |  | 5 8 |  | 5 16 |  |  | 5 32 |  |
| G |  | H |  | I |  |  |  | ^ |  |  |  |
| 5 128 |  | 4 128 |  | 3 128 |  |  |  | 6 8 |  |  |  |
| J |  | K |  | L |  | < |  | v |  |  | > |
| 2 128 |  | 1 128 |  | 0 128 |  | 6 4 |  | 6 2 |  |  | 6 1 |
| M |  | N |  | O |  | P |  |  | DEL |  |  |
| 4 64 |  | 3 64 |  | 2 64 |  | 1 64 |  |  | 0 64 |  |  |
| Q |  | R |  | S |  | T |  |  | U |  |  |
| 4 32 |  | 3 32 |  | 2 32 |  | 1 32 |  |  | 0 32 |  |  |
| V |  | W |  | X |  | Y |  |  | Z |  |  |
| 4 16 |  | 3 16 |  | 2 16 |  | 1 16 |  |  | 0 16 |  |  |
| ALPHA |  | 7 |  | 8 |  | 9 |  |  | * |  |  |
| 7 8 |  | 3 8 |  | 2 8 |  | 1 8 |  |  | 0 8 |  |  |
| <-\ |  | 4 |  | 5 |  | 6 |  |  | - |  |  |
| 7 4 |  | 3 4 |  | 2 4 |  | 1 4 |  |  | 0 4 |  |  |
| /-.> |  | 1 |  | 2 |  | 3 |  |  | + |  |  |
| 7 2 |  | 3 2 |  | 2 2 |  | 1 2 |  |  | 0 2 |  |  |
| ON |  | 0 |  | . |  | SPC |  |  | ENTER |  |  |
| 0 32768 |  | 3 1 |  | 2 1 |  | 1 1 |  |  | 0 1 |  |  |

Scancode

The Scancode defines what the emulator will do when a certain key on the keyboard is pressed. Pressing a key that has no scancode defined will (only if debug mode is on) display a message box with the key's scancode number, so that you can write a Scancode block. The commands in the Scancode block are executed twice, first when the key is pressed and a second time when the key is released. Flags can help to transfer information (for example can be set to show that a shift key is pressed) from one Scancode block to another. Valid flags are 0-31.

Scancode INTEGER

<COMMANDS>

End

Valid Commands

Map INTEGER INTEGER

or

Press INTEGER

or

Release INTEGER

or

IfPressed INTEGER

<COMMANDS>

Else

<COMMANDS>

End

or

SetFlag INTEGER

or

ResetFlag INTEGER

or

NotFlag INTEGER

or

IfFlag INTEGER

<COMMANDS>

Else

<COMMANDS>

End

or

MenuItem INTEGER

Map <SCANCODE> <BUTTON> will map the state (pressed or released) of the key <SCANCODE> (defined by the "Scancode <SCANCODE>" block) to the button <BUTTON> (defined by the "Button <BUTTON>" block). This means when Map is called in the key pressing state the Button is pressed else, when Map is called in the key releasing state, the Button is released.

Example:

Map 51 11

Press <BUTTON> will press the button defined by the "Button <BUTTON>" block.

Example:

Press 11

Release <BUTTON> will release the button defined by the "Button <BUTTON>" block.

Example:

Release 11

IfPressed <SCANCODE> tests if Scancode block is called by a key press or a key release. If the block is called by a key press, the command lines behind IfPressed are executed until a Else or End command is reached. If the block is called by a key release, all command lines between Else and the corresponding End are executed.

Example:

IfPressed 51

Map 51 11

Else

Map 51 12

End

SetFlag <FLAGNO> will set an internal flag.

Example:

SetFlag 1

ResetFlag <FLAGNO> will clear an internal flag.

Example:

ResetFlag 1

NotFlag <FLAGNO> will invert the state of an internal flag.

Example:

NotFlag 1

IfFlag <FLAGNO> tests the state of the flag. If the Flag is set, the command lines behind IfFlag are executed until a Else or End command is reached. If the Flag is not set, all command lines between Else and the corresponding End are executed.

Example:

IfFlag 1

Map 51 11

Else

Map 51 12

End

MenuItem will open a menu (only if it’s supported). The following values are accepted:

FILE_NEW               1

FILE_OPEN              2

FILE_SAVE              3

FILE_SAVEAS            4

FILE_EXIT              5

EDIT_COPY_SCREEN       6

FILE_SETTINGS          7

EDIT_RESET             8

EDIT_LOAD_OBJECT       9

EDIT_SAVE_OBJECT       10

HELP_ABOUT             11

HELP_TOPICS            12

FILE_CLOSE             13

EDIT_BACKUP_SAVE       14

EDIT_BACKUP_RESTORE    15

EDIT_BACKUP_DELETE     16

VIEW_SCRIPT            17

EDIT_COPY_STRING       19

EDIT_PASTE_STRING      20

TOOL_DISASM            21

TOOL_DEBUG             22

TOOL_MACRO_RECORD      23

TOOL_MACRO_PLAY        24

TOOL_MACRO_STOP        25

TOOL_MACRO_SETTINGS    26

Conclusion

This document was originally written by Casey Patterson for the Keyboard Macro Language made by Sébastien Carlier and is now maintained by Christoph Gießelink.

I’m sorry, the authors can’t give any individual support, try the newsgroup comp.sys.hp48 please.

The latest updates are available at:

Christoph Gießelink

c.giesselink@gmx.de

Christoph Giesselink Main Page

http://privat.swol.de/ChristophGiesselink/